ECA5801 - IOE&AI DRIVEN SYSTEM

EXPERIMENT 6

JAYASREE S(192512299)

PROGRAM:

# ================================================================

# PROJECT:

# a. Simulation and Performance Evaluation of People-Process-Data-

#    Things IoE Framework using Edge/Fog-to-Cloud Pipeline with

#    MQTT, Node-RED.

# b. Benchmarking of 5G/6G Communication for IoE Applications

#    based on Throughput, Latency, Reliability and Energy Efficiency.

#

# Environment: Google Colab / Python

# Outputs: P-P-D-T flow, Edge/Fog/Cloud metrics, 5G/6G graphs,

#          MQTT messages, CSV reports and architecture diagram.

#

# NOTE: This is a model-based simulation, not a live 5G/6G test.

# ================================================================

import random

import json

import csv

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

from collections import Counter

# -------------------- 1. SIMULATION SETTINGS --------------------

random.seed(42)

np.random.seed(42)

NUM_PEOPLE = 20

NUM_THINGS = 20

MESSAGES_PER_THING = 50

NUM_MESSAGES = NUM_THINGS * MESSAGES_PER_THING

MQTT_TOPIC = "ioe/ppdt/telemetry"

SIMULATION_TIME_S = 10

PACKET_SIZE_BYTES = 1024

print("=" * 75)

print("IoE P-P-D-T FRAMEWORK AND 5G/6G PERFORMANCE BENCHMARK")

print("=" * 75)

# -------------------- 2. PEOPLE-PROCESS-DATA-THINGS --------------------

people = [

{"person_id": f"P{i:03d}", "role": random.choice(

["Operator", "Engineer", "Supervisor"]

)}

for i in range(1, NUM_PEOPLE + 1)

]

things = [

{

"thing_id": f"T{i:03d}",

"temperature_C": round(random.uniform(20, 40), 2),

"humidity_percent": round(random.uniform(30, 80), 2),

"status": "ACTIVE"

}

for i in range(1, NUM_THINGS + 1)

]

print("\nLAYER 1: PEOPLE")

print("People registered:", len(people))

print("Sample:", people[:5])

print("\nLAYER 2: THINGS")

print("IoE devices registered:", len(things))

print("Sample:", things[:5])

# Process layer: rules for processing sensor information

def process_data(record):

record = record.copy()

record["temperature_alert"] = (

record["temperature_C"] >= 35

)

record["humidity_alert"] = (

record["humidity_percent"] >= 70

)

record["processing_status"] = "PROCESSED"

return record

# Data layer: JSON payload creation

raw_records = []

processed_records = []

for thing in things:

for sequence in range(1, MESSAGES_PER_THING + 1):

raw = {

"person_id": people[

(sequence - 1) % len(people)

]["person_id"],

"thing_id": thing["thing_id"],

"sequence": sequence,

"temperature_C": round(

np.clip(

thing["temperature_C"] + np.random.normal(0, 1),

10, 50

), 2

),

"humidity_percent": round(

np.clip(

thing["humidity_percent"] + np.random.normal(0, 2),

10, 100

), 2

)

}

raw_records.append(raw)

processed_records.append(process_data(raw))

print("\nLAYER 3: PROCESS")

print("Total processed records:", len(processed_records))

print("Temperature alerts:", sum(

r["temperature_alert"] for r in processed_records

))

print("Humidity alerts:", sum(

r["humidity_alert"] for r in processed_records

))

print("\nLAYER 4: DATA")

print("Data format: JSON")

print("MQTT topic:", MQTT_TOPIC)

print("Sample JSON:")

print(json.dumps(processed_records[0], indent=2))

# -------------------- 3. MQTT MESSAGE SIMULATION --------------------

mqtt_messages = []

for record in processed_records:

mqtt_messages.append({

"topic": MQTT_TOPIC,

"qos": 1,

"payload": json.dumps(record)

})

print("\nMQTT MESSAGE SIMULATION")

print("Total messages:", len(mqtt_messages))

print("QoS level: 1")

for message in mqtt_messages[:3]:

print("Topic:", message["topic"])

print("QoS:", message["qos"])

print("Payload:", message["payload"])

# -------------------- 4. NODE-RED-STYLE DATA FLOW --------------------

flow = [

"People / IoE Devices",

"Sensor Data Acquisition",

"MQTT Publish",

"Edge Processing",

"Fog Aggregation",

"Cloud Storage / Analytics",

"Dashboard and Alerts"

]

print("\nNODE-RED-STYLE FLOW")

print(" -> ".join(flow))

# -------------------- 5. EDGE / FOG / CLOUD COMPARISON --------------------

# Assumed model parameters, in milliseconds.

# These are illustrative simulation assumptions, not measured values.

pipeline_parameters = {

"Edge": {

"network_delay_ms": 2,

"processing_delay_ms": 3,

"cloud_transfer_delay_ms": 0,

"energy_per_packet_mJ": 0.8

},

"Fog": {

"network_delay_ms": 8,

"processing_delay_ms": 5,

"cloud_transfer_delay_ms": 0,

"energy_per_packet_mJ": 1.5

},

"Cloud": {

"network_delay_ms": 25,

"processing_delay_ms": 12,

"cloud_transfer_delay_ms": 10,

"energy_per_packet_mJ": 2.5

}

}

pipeline_results = []

for layer, params in pipeline_parameters.items():

latencies = np.maximum(

0.1,

np.random.normal(

params["network_delay_ms"]

+ params["processing_delay_ms"]

+ params["cloud_transfer_delay_ms"],

max(0.5, params["network_delay_ms"] * 0.15),

NUM_MESSAGES

)

)

delivery_rate = {

"Edge": 0.995,

"Fog": 0.990,

"Cloud": 0.980

}[layer]

delivered = np.random.random(NUM_MESSAGES) < delivery_rate

pdr = np.mean(delivered) * 100

delivered_bytes = int(

np.sum(delivered) * PACKET_SIZE_BYTES

)

throughput_mbps = (

delivered_bytes * 8 / SIMULATION_TIME_S / 1_000_000

)

# Energy efficiency: successfully delivered bits / energy in joules

energy_joules = (

np.sum(delivered) * params["energy_per_packet_mJ"] / 1000

)

energy_efficiency = (

delivered_bytes * 8 / energy_joules / 1_000_000

if energy_joules > 0 else 0

)

pipeline_results.append({

"Pipeline": layer,

"Average_Latency_ms": round(np.mean(latencies), 2),

"P95_Latency_ms": round(np.percentile(latencies, 95), 2),

"Throughput_Mbps": round(throughput_mbps, 4),

"Reliability_Percent": round(pdr, 2),

"Energy_Efficiency_Mbit_per_J": round(

energy_efficiency, 4

)

})

pipeline_df = pd.DataFrame(pipeline_results)

print("\nEDGE / FOG / CLOUD PERFORMANCE")

print(pipeline_df.to_string(index=False))

# -------------------- 6. 5G / 6G BENCHMARK --------------------

# Illustrative assumptions for an idealized comparison.

# Energy is modeled as radio/network energy per delivered packet.

network_parameters = {

"5G": {

"mean_latency_ms": 10,

"latency_std_ms": 2.5,

"capacity_mbps": 100,

"reliability": 0.990,

"energy_per_packet_mJ": 1.5

},

"6G": {

"mean_latency_ms": 1,

"latency_std_ms": 0.3,

"capacity_mbps": 1000,

"reliability": 0.999,

"energy_per_packet_mJ": 1.0

}

}

network_results = []

network_latency_samples = {}

for network, params in network_parameters.items():

# Generate packet latency samples.

latency = np.maximum(

0.05,

np.random.normal(

params["mean_latency_ms"],

params["latency_std_ms"],

NUM_MESSAGES

)

)

# Simulate communication reliability.

delivered = np.random.random(NUM_MESSAGES) < params["reliability"]

successful_packets = int(np.sum(delivered))

reliability_percent = successful_packets / NUM_MESSAGES * 100

# Delivered traffic, bounded by the assumed network capacity.

offered_mbps = (

successful_packets * PACKET_SIZE_BYTES * 8

/ SIMULATION_TIME_S / 1_000_000

)

throughput_mbps = min(

offered_mbps, params["capacity_mbps"]

)

# Estimate energy from successfully delivered packets.

energy_joules = (

successful_packets * params["energy_per_packet_mJ"] / 1000

)

total_delivered_bits = (

successful_packets * PACKET_SIZE_BYTES * 8

)

energy_efficiency = (

total_delivered_bits / energy_joules / 1_000_000

if energy_joules > 0 else 0

)

network_latency_samples[network] = latency

network_results.append({

"Network": network,

"Throughput_Mbps": round(throughput_mbps, 4),

"Average_Latency_ms": round(np.mean(latency), 3),

"P95_Latency_ms": round(np.percentile(latency, 95), 3),

"Reliability_Percent": round(reliability_percent, 3),

"Energy_Efficiency_Mbit_per_J": round(

energy_efficiency, 3

)

})

network_df = pd.DataFrame(network_results)

print("\n5G / 6G BENCHMARK RESULTS")

print(network_df.to_string(index=False))

# -------------------- 7. P-P-D-T ARCHITECTURE DIAGRAM --------------------

fig, ax = plt.subplots(figsize=(12, 8))

ax.set_xlim(0, 10)

ax.set_ylim(0, 12)

ax.axis("off")

architecture = [

(10, "PEOPLE", "Operators | Engineers | Supervisors"),

(8, "PROCESS", "Rules | Analytics | Alerts"),

(6, "DATA", "JSON | MQTT | Sensor Measurements"),

(4, "THINGS", "IoE Devices | Sensors | Actuators")

]

for y, title, description in architecture:

ax.text(

5, y, title + "\n" + description,

ha="center", va="center", fontsize=11,

bbox=dict(boxstyle="round,pad=0.7", fill=False, linewidth=1.5)

)

for y1, y2 in [(9.2, 8.8), (7.2, 6.8), (5.2, 4.8)]:

ax.annotate(

"", xy=(5, y2), xytext=(5, y1),

arrowprops=dict(arrowstyle="<->", linewidth=1.5)

)

ax.text(

5, 2,

"Edge Processing -> Fog Aggregation -> Cloud Analytics\n"

"MQTT Communication | Node-RED-Style Flow | Dashboard",

ha="center", va="center", fontsize=10

)

ax.set_title("People-Process-Data-Things IoE Framework")

plt.tight_layout()

plt.savefig("ppdt_architecture.png", dpi=200)

plt.show()

# -------------------- 8. EDGE / FOG / CLOUD GRAPHS --------------------

pipeline_metrics = [

("Average_Latency_ms", "Average Latency (ms)"),

("Throughput_Mbps", "Throughput (Mbps)"),

("Reliability_Percent", "Reliability (%)"),

("Energy_Efficiency_Mbit_per_J", "Energy Efficiency (Mbit/J)")

]

for metric, title in pipeline_metrics:

plt.figure(figsize=(8, 5))

plt.bar(pipeline_df["Pipeline"], pipeline_df[metric])

plt.title("Edge / Fog / Cloud: " + title)

plt.ylabel(title)

plt.xlabel("Processing Pipeline")

plt.grid(axis="y", alpha=0.3)

plt.tight_layout()

plt.savefig(

"pipeline_" + metric.lower() + ".png", dpi=200

)

plt.show()

# -------------------- 9. 5G / 6G COMPARISON GRAPHS --------------------

network_metrics = [

("Throughput_Mbps", "Throughput (Mbps)"),

("Average_Latency_ms", "Average Latency (ms)"),

("Reliability_Percent", "Reliability (%)"),

("Energy_Efficiency_Mbit_per_J", "Energy Efficiency (Mbit/J)")

]

for metric, title in network_metrics:

plt.figure(figsize=(8, 5))

plt.bar(network_df["Network"], network_df[metric])

plt.title("5G vs 6G: " + title)

plt.xlabel("Communication Network")

plt.ylabel(title)

plt.grid(axis="y", alpha=0.3)

plt.tight_layout()

plt.savefig(

"network_" + metric.lower() + ".png", dpi=200

)

plt.show()

# -------------------- 10. LATENCY DISTRIBUTION --------------------

plt.figure(figsize=(9, 5))

for network, samples in network_latency_samples.items():

plt.hist(

samples, bins=25, alpha=0.55,

label=network

)

plt.title("5G / 6G Latency Distribution")

plt.xlabel("Latency (ms)")

plt.ylabel("Packet Count")

plt.legend()

plt.grid(alpha=0.3)

plt.tight_layout()

plt.savefig("5g_6g_latency_distribution.png", dpi=200)

plt.show()

# -------------------- 11. RESULTS TABLE EXPORT --------------------

pipeline_df.to_csv("edge_fog_cloud_results.csv", index=False)

network_df.to_csv("5g_6g_benchmark_results.csv", index=False)

pd.DataFrame(processed_records).to_csv(

"ppdt_processed_data.csv", index=False

)

with open("mqtt_sample_messages.json", "w") as f:

json.dump(mqtt_messages[:20], f, indent=2)

summary = {

"project": "P-P-D-T IoE and 5G/6G Benchmark",

"people_registered": NUM_PEOPLE,

"things_registered": NUM_THINGS,

"messages_generated": NUM_MESSAGES,

"mqtt_topic": MQTT_TOPIC,

"pipeline_results": pipeline_results,

"network_results": network_results,

"simulation_time_seconds": SIMULATION_TIME_S,

"packet_size_bytes": PACKET_SIZE_BYTES,

"model_based_simulation": True

}

with open("ioe_benchmark_summary.json", "w") as f:

json.dump(summary, f, indent=2)

print("\n" + "=" * 75)

print("SIMULATION COMPLETED SUCCESSFULLY")

print("=" * 75)

print("\nOUTPUT FILES")

print("1. ppdt_architecture.png")

print("2. pipeline_average_latency_ms.png")

print("3. pipeline_throughput_mbps.png")

print("4. pipeline_reliability_percent.png")

print("5. pipeline_energy_efficiency_mbit_per_j.png")

print("6. network_throughput_mbps.png")

print("7. network_average_latency_ms.png")

print("8. network_reliability_percent.png")

print("9. network_energy_efficiency_mbit_per_j.png")

print("10. 5g_6g_latency_distribution.png")

print("11. edge_fog_cloud_results.csv")

print("12. 5g_6g_benchmark_results.csv")

print("13. ppdt_processed_data.csv")

print("14. mqtt_sample_messages.json")

print("15. ioe_benchmark_summary.json")

OUTPUT:

===========================================================================

IoE P-P-D-T FRAMEWORK AND 5G/6G PERFORMANCE BENCHMARK

===========================================================================

LAYER 1: PEOPLE

People registered: 20

Sample: [{'person_id': 'P001', 'role': 'Supervisor'}, {'person_id': 'P002', 'role': 'Operator'}, {'person_id': 'P003', 'role': 'Operator'}, {'person_id': 'P004', 'role': 'Supervisor'}, {'person_id': 'P005', 'role': 'Engineer'}]

LAYER 2: THINGS

IoE devices registered: 20

Sample: [{'thing_id': 'T001', 'temperature_C': 24.65, 'humidity_percent': 60.1, 'status': 'ACTIVE'}, {'thing_id': 'T002', 'temperature_C': 31.22, 'humidity_percent': 65.8, 'status': 'ACTIVE'}, {'thing_id': 'T003', 'temperature_C': 34.03, 'humidity_percent': 50.98, 'status': 'ACTIVE'}, {'thing_id': 'T004', 'temperature_C': 28.98, 'humidity_percent': 43.91, 'status': 'ACTIVE'}, {'thing_id': 'T005', 'temperature_C': 37.39, 'humidity_percent': 67.94, 'status': 'ACTIVE'}]

LAYER 3: PROCESS

Total processed records: 1000

Temperature alerts: 223

Humidity alerts: 207

LAYER 4: DATA

Data format: JSON

MQTT topic: ioe/ppdt/telemetry

Sample JSON:

{

"person_id": "P001",

"thing_id": "T001",

"sequence": 1,

"temperature_C": 25.15,

"humidity_percent": 59.82,

"temperature_alert": false,

"humidity_alert": false,

"processing_status": "PROCESSED"

}

MQTT MESSAGE SIMULATION

Total messages: 1000

QoS level: 1

Topic: ioe/ppdt/telemetry

QoS: 1

Payload: {"person_id": "P001", "thing_id": "T001", "sequence": 1, "temperature_C": 25.15, "humidity_percent": 59.82, "temperature_alert": false, "humidity_alert": false, "processing_status": "PROCESSED"}

Topic: ioe/ppdt/telemetry

QoS: 1

Payload: {"person_id": "P002", "thing_id": "T001", "sequence": 2, "temperature_C": 25.3, "humidity_percent": 63.15, "temperature_alert": false, "humidity_alert": false, "processing_status": "PROCESSED"}

Topic: ioe/ppdt/telemetry

QoS: 1

Payload: {"person_id": "P003", "thing_id": "T001", "sequence": 3, "temperature_C": 24.42, "humidity_percent": 59.63, "temperature_alert": false, "humidity_alert": false, "processing_status": "PROCESSED"}

NODE-RED-STYLE FLOW

People / IoE Devices -> Sensor Data Acquisition -> MQTT Publish -> Edge Processing -> Fog Aggregation -> Cloud Storage / Analytics -> Dashboard and Alerts

EDGE / FOG / CLOUD PERFORMANCE

Pipeline  Average_Latency_ms  P95_Latency_ms  Throughput_Mbps  Reliability_Percent  Energy_Efficiency_Mbit_per_J

Edge                5.00            5.81           0.8151                 99.5                       10.2400

Fog               12.93           14.77           0.8135                 99.3                        5.4613

Cloud               46.85           52.92           0.8077                 98.6                        3.2768

5G / 6G BENCHMARK RESULTS

Network  Throughput_Mbps  Average_Latency_ms  P95_Latency_ms  Reliability_Percent  Energy_Efficiency_Mbit_per_J

5G           0.8143              10.029          14.308                 99.4                         5.461

6G           0.8192               0.989           1.464                100.0                         8.192

==========================================================================

SIMULATION COMPLETED SUCCESSFULLY

===========================================================================